- CIS 83 Case Study

- December 3, 2006

Midnight Hackers Team:

- Rich Simms

- Christian Paasche

- Jay Clark

- Salvador Marquez

- Sean Lazar

- Mo Hart

Scenario:

Corporate LAN contains a router connected to an ISP via Frame Relay. Corporate router does PAT and DHCP for Corporate LAN. Access and Distribution switches are configured with VLANs, trunking, and port security. The ISP router is connected to two branch office routers via serial links, using PPP and authenticated via CHAP. Configure a dynamic routing protocol on each of the three ISP routers. Each of the three ISP routers has a LAN attached.

Objective:

Determine an IP address scheme that fits this model. Configure serial links for ISP, Branch Office 1 (BR_1), and Branch Office 2 (BR_2) with PPP and CHAP authentication. Use /30 subnets for the serial links. Write configurations for both OSPF and EIGRP, to gain experience with both protocols. Default traffic needs to be sent to Loopback 100 on Branch Office 2. Branch Office 1 and Branch Office 2 have LANs, and need to be configured with VLSM networks that are part of 172.16.0.0.

On the Corporate router, it should be the default gateway, DHCP server, and PAT for the Corporate LAN. The Corporate LAN needs to be configured with RFC 1918 (private addressing). The Access and Distribution switches need to be configured with VLANs 1, 10, 20 and 99. Each switch must have one port on each VLAN. All ports are manually configured as access or trunk ports. Port security needs to be configured on all access ports to allow only 1 MAC address. STP Root Bridge is the Distribution switch and it needs to be configured as such.  A discard route is also placed on the corporate router. A discard route prevents packets intended for the corporate VLANs to be sent out the default gateway in the event one of the corporate LANs becomes unreachable.

Network Diagram:

Physical Cabling:

Hardware:

BR_1 Cisco 2620 Router with 1 two port serial card, IOS version 12.2
BR_2 Cisco 2620 Router with 1 two port serial card, IOS version 12.2
ISP Cisco 2621 Router with 2 two port serial cards, IOS version 12.2

Corporate Cisco 2620 Router with 1 two port serial card and 1 ISDN card, IOS version 12.2
Distribution Cisco 2950 Switch with 24 Fast Ethernet ports and 2 Gigabit Copper ports, IOS version 12.1
Access Cisco 2900 Switch with 12 Fast Ethernet ports, IOS version 12.0

Corporate LAN VLANs:

|  | VLAN | Network | Subnet Mask |
| --- | --- | --- | --- |
| Default | 1 | 192.168.10.0 | 255.255.255.224 |
| Accounting | 10 | 192.168.10.32 | 255.255.255.224 |
| Marketing | 20 | 192.168.10.64 | 255.255.255.240 |
| Engineering | 99 | 192.168.10.80 | 255.255.255.240 |

VLAN Port Assignment:

|  | Trunk | VLAN 1 | VLAN 10 | VLAN 20 | VLAN 99 |
| --- | --- | --- | --- | --- | --- |
| Distribution | Gig 0/1 & 0/2 | Fa 1-6 | Fa 7-12 | Fa 13-18 | Fa 19-24 |
| Access | Fa 1 | Fa 2-3 | Fa 4-6 | Fa 7-9 | Fa 10-12 |

# Running Configurations

# We implemented this case study using both EIGRP and OSPF routing protocols.  The running configurations below are labeled OSPF for the OSPF implementation and EIGRP for the EIGRP implementation.  Note the actual change in routing protocols is only done to the isp, br_1 and br_2 routers.  The running configurations for the corporate router and the two switches are the same in both implementations.

# OSPF Running Configurations:

| ISP 11/22/06 (OSPF) | BR_1 11/22/06 (OSPF) | BR_2 11/22/06 (OSPF) |
| --- | --- | --- |
| Current configuration : 1255 bytes <output omitted> ! hostname isp ! enable secret 5 $1$PjZ2$S3t6XejUQpH4kRmEJgK6I0 ! username br_1 password 0 cisco username br_2 password 0 cisco ip subnet-zero ! no ip domain-lookup ! <output omitted> ! interface FastEthernet0/0 ip address 63.88.27.129 255.255.255.128 duplex auto speed auto ! interface Serial0/0 ip address 63.88.27.65 255.255.255.252 encapsulation frame-relay frame-relay map ip 63.88.27.66 201 ! interface FastEthernet0/1 no ip address shutdown duplex auto speed auto ! interface Serial0/1 ip address 207.62.41.25 255.255.255.252 encapsulation ppp ppp authentication chap ! interface Serial0/2 no ip address shutdown ! interface Serial0/3 ip address 207.62.41.21 255.255.255.252 encapsulation ppp ppp authentication chap ! router ospf 10 log-adjacency-changes passive-interface FastEthernet0/0 network 63.88.27.64 0.0.0.3 area 0 network 63.88.27.128 0.0.0.127 area 0 network 207.62.41.20 0.0.0.3 area 0 network 207.62.41.24 0.0.0.3 area 0 ! ip classless ip http server ! ! dial-peer cor custom ! ! ! ! line con 0 exec-timeout 0 0 logging synchronous line aux 0 line vty 0 4 password cisco login ! end | Current configuration : 1074 bytes <output omitted> ! hostname br_1 ! enable secret 5 $1$S3ry$Gh2CqXLrXefkJEjSkO6/r0 ! username br_2 password 0 cisco username isp password 0 cisco memory-size iomem 15 ip subnet-zero ! no ip domain-lookup ! <output omitted> ! interface FastEthernet0/0 ip address 172.16.0.129 255.255.255.192 duplex auto speed auto ! interface Serial0/0 ip address 207.62.41.29 255.255.255.252 encapsulation ppp no fair-queue ppp authentication chap ! interface Serial0/1 ip address 207.62.41.22 255.255.255.252 encapsulation ppp clockrate 64000 ppp authentication chap ! router ospf 10 log-adjacency-changes passive-interface FastEthernet0/0 network 172.16.0.128 0.0.0.63 area 0 network 207.62.41.20 0.0.0.3 area 0 network 207.62.41.28 0.0.0.3 area 0 ! ip classless ip http server ! ! voice-port 1/0/0 ! voice-port 1/0/1 ! dial-peer cor custom ! ! ! ! line con 0 exec-timeout 0 0 logging synchronous line aux 0 line vty 0 4 password cisco login ! end | Current configuration : 1196 bytes <output omitted> ! hostname br_2 ! enable secret 5 $1$9vG0$GA5f0sPbCgyyELNRWssmj1 ! username br_1 password 0 cisco username isp password 0 cisco memory-size iomem 10 ip subnet-zero ! no ip domain-lookup ! <output omitted> ! interface Loopback100 ip address 10.10.10.10 255.255.255.252 ! interface FastEthernet0/0 ip address 172.16.0.1 255.255.255.128 duplex auto speed auto ! interface Serial0/0 ip address 207.62.41.30 255.255.255.252 encapsulation ppp clockrate 64000 ppp authentication chap ! interface Serial0/1 ip address 207.62.41.26 255.255.255.252 encapsulation ppp clockrate 64000 ppp authentication chap ! router ospf 10 log-adjacency-changes passive-interface FastEthernet0/0 network 172.16.0.0 0.0.0.127 area 0 network 207.62.41.24 0.0.0.3 area 0 network 207.62.41.28 0.0.0.3 area 0 default-information originate ! ip classless ip route 0.0.0.0 0.0.0.0 Loopback100 ip http server ! ! ! voice-port 1/0/0 ! voice-port 1/0/1 ! dial-peer cor custom ! ! ! ! ! line con 0 exec-timeout 0 0 logging synchronous line aux 0 line vty 0 4 password cisco login ! end |

| Corporate 11/22/06 (OSPF) | Distribution 11/22/06 (OSPF) | Access 11/22/06 (OSPF) |
| --- | --- | --- |
| Current configuration : 2112 bytes <output omitted> ! hostname corporate ! enable secret 5 $1$p3Ob$4EGMu.8vfqTRzz/OBQOMV/ ! memory-size iomem 15 ip subnet-zero ! no ip domain-lookup ip dhcp excluded-address 192.168.10.1 192.168.10.3 ip dhcp excluded-address 192.168.10.33 ip dhcp excluded-address 192.168.10.65 ip dhcp excluded-address 192.168.10.81 ! ip dhcp pool vlan1 network 192.168.10.0 255.255.255.224 default-router 192.168.10.1 ! ip dhcp pool vlan10 network 192.168.10.32 255.255.255.224 default-router 192.168.10.33 ! ip dhcp pool vlan20 network 192.168.10.64 255.255.255.240 default-router 192.168.10.65 ! ip dhcp pool vlan99 network 192.168.10.80 255.255.255.240 default-router 192.168.10.81 ! <output omitted> ! interface FastEthernet0/0 no ip address duplex auto speed auto ! interface FastEthernet0/0.1 encapsulation dot1Q 1 native ip address 192.168.10.1 255.255.255.224 ip nat inside ! interface FastEthernet0/0.10 encapsulation dot1Q 10 ip address 192.168.10.33 255.255.255.224 ip nat inside ! interface FastEthernet0/0.20 encapsulation dot1Q 20 ip address 192.168.10.65 255.255.255.240 ip nat inside ! interface FastEthernet0/0.99 encapsulation dot1Q 99 ip address 192.168.10.81 255.255.255.240 ip nat inside ! interface Serial0/0 ip address 63.88.27.66 255.255.255.252 ip nat outside encapsulation frame-relay frame-relay map ip 63.88.27.65 102 ! <output omitted> ! ip nat inside source list 1 interface Serial0/0 overload ip classless ip route 0.0.0.0 0.0.0.0 63.88.27.65 ip route 192.168.0.0 255.255.0.0 Null0 ip http server ! access-list 1 permit 192.168.10.0 0.0.0.31 access-list 1 permit 192.168.10.32 0.0.0.31 access-list 1 permit 192.168.10.64 0.0.0.15 access-list 1 permit 192.168.10.80 0.0.0.15 ! <output omitted> ! line con 0 exec-timeout 0 0 logging synchronous line aux 0 line vty 0 4 password cisco login ! end | Current configuration : 2766 bytes <output omitted> ! hostname distribution ! enable secret 5 $1$KU5Z$260U8/CK9RlIChyPOTzRY0 ! ip subnet-zero no ip domain-lookup ! spanning-tree extend system-id spanning-tree vlan 1 priority 24576 ! ! interface FastEthernet0/1 switchport mode access switchport port-security no ip address ! <interfaces 2-6 like FastEthernet0/1> ! interface FastEthernet0/7 switchport access vlan 10 switchport mode access switchport port-security no ip address ! <interfaces 8-12 like FastEthernet0/7> ! interface FastEthernet0/13 switchport access vlan 20 switchport mode access switchport port-security no ip address ! <interfaces 14-18 like FastEthernet0/13> ! interface FastEthernet0/19 switchport access vlan 99 switchport mode access switchport port-security no ip address ! <interfaces 20-24 like FastEthernet0/19> ! interface GigabitEthernet0/1 switchport mode trunk no ip address ! interface GigabitEthernet0/2 switchport mode trunk no ip address ! interface Vlan1 ip address 192.168.10.2 255.255.255.224 no ip route-cache ! ip http server ! ! line con 0 exec-timeout 0 0 logging synchronous line vty 0 4 password cisco login line vty 5 15 login ! end | Current configuration: <output omitted> ! hostname access ! enable secret 5 $1$NKNh$qEBH.dDHIzWCvQ5nszU6/. ! ip subnet-zero no ip domain-lookup ! interface FastEthernet0/1 switchport trunk encapsulation dot1q switchport mode trunk ! interface FastEthernet0/2 port security max-mac-count 1 ! interface FastEthernet0/3 port security max-mac-count 1 ! interface FastEthernet0/4 port security max-mac-count 1 switchport access vlan 10 ! interface FastEthernet0/5 port security max-mac-count 1 switchport access vlan 10 ! interface FastEthernet0/6 port security max-mac-count 1 switchport access vlan 10 ! interface FastEthernet0/7 port security max-mac-count 1 switchport access vlan 20 ! interface FastEthernet0/8 port security max-mac-count 1 switchport access vlan 20 ! interface FastEthernet0/9 port security max-mac-count 1 switchport access vlan 20 ! interface FastEthernet0/10 port security max-mac-count 1 switchport access vlan 99 ! interface FastEthernet0/11 port security max-mac-count 1 switchport access vlan 99 ! interface FastEthernet0/12 port security max-mac-count 1 switchport access vlan 99 ! interface VLAN1 ip address 192.168.10.3 255.255.255.224 no ip directed-broadcast no ip route-cache ! mac-address-table secure 0002.b34b.a16f FastEthernet0/2 vlan 1 mac-address-table secure 0002.b34c.2383 FastEthernet0/7 vlan 20 mac-address-table secure 0002.b34c.33aa FastEthernet0/10 vlan 99 mac-address-table secure 0002.b35d.e8bf FastEthernet0/4 vlan 10 ! line con 0 exec-timeout 0 0 logging synchronous transport input none stopbits 1 line vty 0 4 password cisco login line vty 5 15 login ! end |

# OSPF Routing tables:

Corporate Router (OSPF)

Codes: C - connected, S - static, I - IGRP, R - RIP, M - mobile, B - BGP D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2 E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area * - candidate default, U - per-user static route, o - ODR P - periodic downloaded static route Gateway of last resort is 63.88.27.65 to network 0.0.0.0 192.168.10.0/24 is variably subnetted, 4 subnets, 2 masks C 192.168.10.64/28 is directly connected, FastEthernet0/0.20 C 192.168.10.80/28 is directly connected, FastEthernet0/0.99 C 192.168.10.32/27 is directly connected, FastEthernet0/0.10 C 192.168.10.0/27 is directly connected, FastEthernet0/0.1 63.0.0.0/30 is subnetted, 1 subnets C 63.88.27.64 is directly connected, Serial0/0 S* 0.0.0.0/0 [1/0] via 63.88.27.65 S 192.168.0.0/16 is directly connected, Null0

ISP Router (OSPF)

Codes: C - connected, S - static, I - IGRP, R - RIP, M - mobile, B - BGP D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2 E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2 ia - IS-IS inter area, * - candidate default, U - per-user static route o - ODR, P - periodic downloaded static route Gateway of last resort is 207.62.41.26 to network 0.0.0.0 172.16.0.0/16 is variably subnetted, 2 subnets, 2 masks O 172.16.0.128/26 [110/65] via 207.62.41.22, 00:23:43, Serial0/3 O 172.16.0.0/25 [110/65] via 207.62.41.26, 00:23:43, Serial0/1 207.62.41.0/24 is variably subnetted, 5 subnets, 2 masks C 207.62.41.24/30 is directly connected, Serial0/1 C 207.62.41.26/32 is directly connected, Serial0/1 O 207.62.41.28/30 [110/845] via 207.62.41.22, 00:23:43, Serial0/3 [110/845] via 207.62.41.26, 00:23:43, Serial0/1 C 207.62.41.20/30 is directly connected, Serial0/3 C 207.62.41.22/32 is directly connected, Serial0/3 63.0.0.0/8 is variably subnetted, 2 subnets, 2 masks C 63.88.27.64/30 is directly connected, Serial0/0 C 63.88.27.128/25 is directly connected, FastEthernet0/0 O*E2 0.0.0.0/0 [110/1] via 207.62.41.26, 00:23:46, Serial0/1

# BR_1 Router (OSPF)

# Codes: C - connected, S - static, I - IGRP, R - RIP, M - mobile, B - BGP # D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area # N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2 # E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP # i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2 # ia - IS-IS inter area, * - candidate default, U - per-user static route # o - ODR, P - periodic downloaded static route # Gateway of last resort is 207.62.41.30 to network 0.0.0.0 # 172.16.0.0/16 is variably subnetted, 2 subnets, 2 masks # C 172.16.0.128/26 is directly connected, FastEthernet0/0 # O 172.16.0.0/25 [110/782] via 207.62.41.30, 00:26:34, Serial0/0 # 207.62.41.0/24 is variably subnetted, 5 subnets, 2 masks # O 207.62.41.24/30 [110/845] via 207.62.41.21, 00:26:34, Serial0/1 # C 207.62.41.28/30 is directly connected, Serial0/0 # C 207.62.41.30/32 is directly connected, Serial0/0 # C 207.62.41.20/30 is directly connected, Serial0/1 # C 207.62.41.21/32 is directly connected, Serial0/1 # 63.0.0.0/8 is variably subnetted, 2 subnets, 2 masks # O 63.88.27.64/30 [110/845] via 207.62.41.21, 00:26:34, Serial0/1 # O 63.88.27.128/25 [110/782] via 207.62.41.21, 00:26:34, Serial0/1 # O*E2 0.0.0.0/0 [110/1] via 207.62.41.30, 00:26:34, Serial0/0

# BR_2 Router (OSPF)

# Codes: C - connected, S - static, I - IGRP, R - RIP, M - mobile, B - BGP # D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area # N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2 # E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP # i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area # * - candidate default, U - per-user static route, o - ODR # P - periodic downloaded static route # Gateway of last resort is 0.0.0.0 to network 0.0.0.0 # 172.16.0.0/16 is variably subnetted, 2 subnets, 2 masks # O 172.16.0.128/26 [110/782] via 207.62.41.29, 00:24:57, Serial0/0 # C 172.16.0.0/25 is directly connected, FastEthernet0/0 # 207.62.41.0/24 is variably subnetted, 5 subnets, 2 masks # C 207.62.41.24/30 is directly connected, Serial0/1 # C 207.62.41.25/32 is directly connected, Serial0/1 # C 207.62.41.28/30 is directly connected, Serial0/0 # C 207.62.41.29/32 is directly connected, Serial0/0 # O 207.62.41.20/30 [110/845] via 207.62.41.25, 00:24:58, Serial0/1 # 10.0.0.0/30 is subnetted, 1 subnets # C 10.10.10.8 is directly connected, Loopback100 # 63.0.0.0/8 is variably subnetted, 2 subnets, 2 masks # O 63.88.27.64/30 [110/845] via 207.62.41.25, 00:24:58, Serial0/1 # O 63.88.27.128/25 [110/782] via 207.62.41.25, 00:24:59, Serial0/1 # S* 0.0.0.0/0 is directly connected, Loopback100

# Corporate Outputs

# corporate#show ip int brief # Interface IP-Address OK? Method Status Prot # ocol # FastEthernet0/0 unassigned YES unset up up # FastEthernet0/0.1 192.168.10.1 YES manual up up # FastEthernet0/0.10 192.168.10.33 YES manual up up # FastEthernet0/0.20 192.168.10.65 YES manual up up # FastEthernet0/0.99 192.168.10.81 YES manual up up # Serial0/0 63.88.27.66 YES manual up up # BRI0/0 unassigned YES unset administratively down down # BRI0/0:1 unassigned YES unset administratively down down # BRI0/0:2 unassigned YES unset administratively down down # FastEthernet0/1 unassigned YES unset administratively down down # Serial0/1 unassigned YES unset administratively down down # corporate#show frame-relay map # Serial0/0 (up): ip 0.0.0.0 dlci 104(0x68,0x1880) # broadcast, # CISCO, status defined, inactive # Serial0/0 (up): ip 0.0.0.0 dlci 103(0x67,0x1870) # broadcast, # CISCO, status defined, inactive # Serial0/0 (up): ip 63.88.27.65 dlci 102(0x66,0x1860), static, # CISCO, status defined, active # corporate#show ip dhcp binding # IP address Client-ID/ Lease expiration Type # Hardware address # 192.168.10.34 0100.02b3.5de8.bf Mar 02 1993 12:18 AM Automatic # 192.168.10.66 0100.02b3.4c23.83 Mar 02 1993 12:19 AM Automatic # 192.168.10.82 0100.02b3.4c33.aa Mar 02 1993 12:19 AM Automatic # corporate#show ip nat statistics # Total active translations: 0 (0 static, 0 dynamic; 0 extended) # Outside interfaces: # Serial0/0 # Inside interfaces: # FastEthernet0/0.1, FastEthernet0/0.10, FastEthernet0/0.20 # FastEthernet0/0.99 # Hits: 531 Misses: 67 # Expired translations: 67 # Dynamic mappings: # -- Inside Source # [Id: 1] access-list 1 interface Serial0/0 refcount 0

# Access Outputs

# access#show vlan # VLAN Name Status Ports # ---- -------------------------------- --------- ------------------------------- # 1 default active Fa0/2, Fa0/3 # 10 accounting active Fa0/4, Fa0/5, Fa0/6 # 20 marketing active Fa0/7, Fa0/8, Fa0/9 # 99 engineering active Fa0/10, Fa0/11, Fa0/12 # 1002 fddi-default active # 1003 token-ring-default active # 1004 fddinet-default active # 1005 trnet-default active # VLAN Type SAID MTU Parent RingNo BridgeNo Stp BrdgMode Trans1 Trans2 # ---- ----- ---------- ----- ------ ------ -------- ---- -------- ------ ------ # 1 enet 100001 1500 - - - - - 0 0 # 10 enet 100010 1500 - - - - - 0 0 # 20 enet 100020 1500 - - - - - 0 0 # 99 enet 100099 1500 - - - - - 0 0 # 1002 fddi 101002 1500 - 0 - - - 0 0 # 1003 tr 101003 1500 - 0 - - srb 0 0 # 1004 fdnet 101004 1500 - - - ieee - 0 0 # 1005 trnet 101005 1500 - - - ibm - 0 0 # access#show spanning-tree # Spanning tree 1 is executing the IEEE compatible Spanning Tree protocol # Bridge Identifier has priority 32768, address 0003.e334.a0c0 # Configured hello time 2, max age 20, forward delay 15 # Current root has priority 24577, address 000b.fc28.d400 # Root port is 13, cost of root path is 19 # Topology change flag not set, detected flag not set, changes 21 # Times: hold 1, topology change 35, notification 2 # hello 2, max age 20, forward delay 15 # Timers: hello 0, topology change 0, notification 0 # Interface Fa0/1 (port 13) in Spanning tree 1 is FORWARDING # Port path cost 19, Port priority 128 # Designated root has priority 24577, address 000b.fc28.d400 # Designated bridge has priority 24577, address 000b.fc28.d400 # Designated port is 25, path cost 0 # Timers: message age 2, forward delay 0, hold 0 # BPDU: sent 6, received 8667 # Interface Fa0/2 (port 14) in Spanning tree 1 is FORWARDING # Port path cost 19, Port priority 128 # Designated root has priority 24577, address 000b.fc28.d400 # Designated bridge has priority 32768, address 0003.e334.a0c0 # Designated port is 14, path cost 19 # Timers: message age 0, forward delay 0, hold 0 # BPDU: sent 8695, received 0 # Interface Fa0/3 (port 15) in Spanning tree 1 is down # Port path cost 100, Port priority 128 # Designated root has priority 24577, address 000b.fc28.d400 # Designated bridge has priority 32768, address 0003.e334.a0c0 # Designated port is 15, path cost 19 # Timers: message age 0, forward delay 0, hold 0 # BPDU: sent 2, received 0 # access#show int fa 0/1 switchport # Name: Fa0/1 # Switchport: Enabled # Administrative mode: trunk # Operational Mode: trunk # Administrative Trunking Encapsulation: dot1q # Operational Trunking Encapsulation: dot1q # Negotiation of Trunking: Disabled # Access Mode VLAN: 0 ((Inactive)) # Trunking Native Mode VLAN: 1 (default) # Trunking VLANs Enabled: ALL # Trunking VLANs Active: 1,10,20,99 # Pruning VLANs Enabled: 2-1001 # Priority for untagged frames: 0 # Override vlan tag priority: FALSE # Voice VLAN: none # Appliance trust: none

# Distribution Outputs

# distribution#show vlan # VLAN Name Status Ports # ---- -------------------------------- --------- ------------------------------- # 1 default active Fa0/1, Fa0/2, Fa0/3, Fa0/4 # Fa0/5, Fa0/6 # 10 accounting active Fa0/7, Fa0/8, Fa0/9, Fa0/10 # Fa0/11, Fa0/12 # 20 marketing active Fa0/13, Fa0/14, Fa0/15, Fa0/16 # Fa0/17, Fa0/18 # 99 engineering active Fa0/19, Fa0/20, Fa0/21, Fa0/22 # Fa0/23, Fa0/24 # 1002 fddi-default active # 1003 token-ring-default active # 1004 fddinet-default active # 1005 trnet-default active # VLAN Type SAID MTU Parent RingNo BridgeNo Stp BrdgMode Trans1 Trans2 # ---- ----- ---------- ----- ------ ------ -------- ---- -------- ------ ------ # 1 enet 100001 1500 - - - - - 0 0 # 10 enet 100010 1500 - - - - - 0 0 # 20 enet 100020 1500 - - - - - 0 0 # 99 enet 100099 1500 - - - - - 0 0 # 1002 fddi 101002 1500 - - - - - 0 0 # VLAN Type SAID MTU Parent RingNo BridgeNo Stp BrdgMode Trans1 Trans2 # ---- ----- ---------- ----- ------ ------ -------- ---- -------- ------ ------ # 1003 tr 101003 1500 - - - - - 0 0 # 1004 fdnet 101004 1500 - - - ieee - 0 0 # 1005 trnet 101005 1500 - - - ibm - 0 0 # Remote SPAN VLANs # ------------------------------------------------------------------------------ # Primary Secondary Type Ports # ------- --------- ----------------- ------------------------------------------ # distribution#show spanning-tree # VLAN0001 # Spanning tree enabled protocol ieee # Root ID Priority 24577 # Address 000b.fc28.d400 # This bridge is the root # Hello Time 2 sec Max Age 20 sec Forward Delay 15 sec # Bridge ID Priority 24577 (priority 24576 sys-id-ext 1) # Address 000b.fc28.d400 # Hello Time 2 sec Max Age 20 sec Forward Delay 15 sec # Aging Time 300 # Interface Port ID Designated Port ID # Name Prio.Nbr Cost Sts Cost Bridge ID Prio.Nbr # ---------------- -------- --------- --- --------- -------------------- -------- # Fa0/1 128.1 19 FWD 0 24577 000b.fc28.d400 128.1 # Fa0/3 128.3 19 FWD 0 24577 000b.fc28.d400 128.3 # Fa0/4 128.4 19 FWD 0 24577 000b.fc28.d400 128.4 # Fa0/5 128.5 19 FWD 0 24577 000b.fc28.d400 128.5 # Fa0/6 128.6 19 FWD 0 24577 000b.fc28.d400 128.6 # Gi0/1 128.25 19 FWD 0 24577 000b.fc28.d400 128.25 # Gi0/2 128.26 19 FWD 0 24577 000b.fc28.d400 128.26 # VLAN0010 # Spanning tree enabled protocol ieee # Root ID Priority 32768 # Address 0003.e334.a0c1 # Cost 19 # Port 25 (GigabitEthernet0/1) # Hello Time 2 sec Max Age 20 sec Forward Delay 15 sec # Bridge ID Priority 32778 (priority 32768 sys-id-ext 10) # Address 000b.fc28.d400 # Hello Time 2 sec Max Age 20 sec Forward Delay 15 sec # Aging Time 300 # Interface Port ID Designated Port ID # Name Prio.Nbr Cost Sts Cost Bridge ID Prio.Nbr # ---------------- -------- --------- --- --------- -------------------- -------- # Gi0/1 128.25 19 FWD 0 32768 0003.e334.a0c1 128.13 # Gi0/2 128.26 19 FWD 19 32778 000b.fc28.d400 128.26 # VLAN0020 # Spanning tree enabled protocol ieee # Root ID Priority 32768 # Address 0003.e334.a0c2 # Cost 19 # Port 25 (GigabitEthernet0/1) # Hello Time 2 sec Max Age 20 sec Forward Delay 15 sec # Bridge ID Priority 32788 (priority 32768 sys-id-ext 20) # Address 000b.fc28.d400 # Hello Time 2 sec Max Age 20 sec Forward Delay 15 sec # Aging Time 300 # Interface Port ID Designated Port ID # Name Prio.Nbr Cost Sts Cost Bridge ID Prio.Nbr # ---------------- -------- --------- --- --------- -------------------- -------- # Gi0/1 128.25 19 FWD 0 32768 0003.e334.a0c2 128.13 # Gi0/2 128.26 19 FWD 19 32788 000b.fc28.d400 128.26 # VLAN0099 # Spanning tree enabled protocol ieee # Root ID Priority 32768 # Address 0003.e334.a0c3 # Cost 19 # Port 25 (GigabitEthernet0/1) # Hello Time 2 sec Max Age 20 sec Forward Delay 15 sec # Bridge ID Priority 32867 (priority 32768 sys-id-ext 99) # Address 000b.fc28.d400 # Hello Time 2 sec Max Age 20 sec Forward Delay 15 sec # Aging Time 300 # Interface Port ID Designated Port ID # Name Prio.Nbr Cost Sts Cost Bridge ID Prio.Nbr # ---------------- -------- --------- --- --------- -------------------- -------- # Gi0/1 128.25 19 FWD 0 32768 0003.e334.a0c3 128.13 # Gi0/2 128.26 19 FWD 19 32867 000b.fc28.d400 128.26 # distribution# show int gi 0/1 switchport # Name: Gi0/1 # Switchport: Enabled # Administrative Mode: trunk # Operational Mode: trunk # Administrative Trunking Encapsulation: dot1q # Operational Trunking Encapsulation: dot1q # Negotiation of Trunking: On # Access Mode VLAN: 1 (default) # Trunking Native Mode VLAN: 1 (default) # Administrative private-vlan host-association: none # Administrative private-vlan mapping: none # Operational private-vlan: none # Trunking VLANs Enabled: ALL # Pruning VLANs Enabled: 2-1001 # Protected: false # Voice VLAN: none (Inactive) # Appliance trust: none # distribution# show int gi 0/2 switchport # Name: Gi0/2 # Switchport: Enabled # Administrative Mode: trunk # Operational Mode: trunk # Administrative Trunking Encapsulation: dot1q # Operational Trunking Encapsulation: dot1q # Negotiation of Trunking: On # Access Mode VLAN: 1 (default) # Trunking Native Mode VLAN: 1 (default) # Administrative private-vlan host-association: none # Administrative private-vlan mapping: none # Operational private-vlan: none # Trunking VLANs Enabled: ALL # Pruning VLANs Enabled: 2-1001 # Protected: false # Voice VLAN: none (Inactive) # Appliance trust: none

# EIGRP Running Configurations:

| ISP 11/22/06 (EIGRP) | BR_1 11/22/06 (EIGRP) | BR_2 11/22/06 (EIGRP) |
| --- | --- | --- |
| Current configuration : 1237 bytes <output omitted> ! hostname isp ! enable secret 5 $1$3hV6$gLtq9ql4ACLDRZ.lr5sD50 ! username br_1 password 0 cisco username br_2 password 0 cisco ip subnet-zero ! ! no ip domain-lookup ! <output omitted> ! interface FastEthernet0/0 ip address 63.88.27.129 255.255.255.128 duplex auto speed auto ! interface Serial0/0 ip address 63.88.27.65 255.255.255.252 encapsulation frame-relay frame-relay map ip 63.88.27.66 201 ! interface FastEthernet0/1 no ip address shutdown duplex auto speed auto ! interface Serial0/1 ip address 207.62.41.25 255.255.255.252 encapsulation ppp ppp authentication chap ! interface Serial0/2 no ip address shutdown ! interface Serial0/3 ip address 207.62.41.21 255.255.255.252 encapsulation ppp ppp authentication chap ! router eigrp 1 redistribute connected redistribute static passive-interface FastEthernet0/0 network 63.88.27.128 0.0.0.127 network 207.62.41.20 0.0.0.3 network 207.62.41.24 0.0.0.3 no auto-summary ! ip classless ip http server ! <output omitted> ! line con 0 exec-timeout 0 0 logging synchronous line aux 0 line vty 0 4 password cisco login ! End | Current configuration : 1017 bytes <output omitted> ! hostname br_1 ! enable secret 5 $1$2F6h$lkPUjSNTHp44uCTXawKKh. ! username br_2 password 0 cisco username isp password 0 cisco memory-size iomem 15 ip subnet-zero ! no ip domain-lookup ! <output omitted> ! interface FastEthernet0/0 ip address 172.16.0.129 255.255.255.192 duplex auto speed auto ! interface Serial0/0 ip address 207.62.41.29 255.255.255.252 encapsulation ppp ppp authentication chap ! interface Serial0/1 ip address 207.62.41.22 255.255.255.252 encapsulation ppp clockrate 64000 ppp authentication chap ! router eigrp 1 passive-interface FastEthernet0/0 network 172.16.0.128 0.0.0.63 network 207.62.41.20 0.0.0.3 network 207.62.41.28 0.0.0.3 no auto-summary ! ip classless ip http server ! <output omitted> ! line con 0 exec-timeout 0 0 logging synchronous line aux 0 line vty 0 4 password cisco login ! end | Current configuration : 1189 bytes <output omitted> ! hostname br_2 ! enable secret 5 $1$Vqwq$lE8e5tenjg47DNIgBrPVT0 ! username br_1 password 0 cisco username isp password 0 cisco memory-size iomem 15 ip subnet-zero ! no ip domain-lookup ! <output omitted> ! interface Loopback100 ip address 10.10.10.10 255.255.255.252 ! interface FastEthernet0/0 ip address 172.16.0.1 255.255.255.128 duplex auto speed auto ! interface Serial0/0 ip address 207.62.41.30 255.255.255.252 encapsulation ppp no fair-queue clockrate 64000 ppp authentication chap ! interface Serial0/1 ip address 207.62.41.26 255.255.255.252 encapsulation ppp clockrate 64000 ppp authentication chap ! router eigrp 1 redistribute static passive-interface FastEthernet0/0 network 172.16.0.0 0.0.0.127 network 207.62.41.24 0.0.0.3 network 207.62.41.28 0.0.0.3 no auto-summary ! ip classless ip route 0.0.0.0 0.0.0.0 Loopback100 ip http server ! <output omitted> ! line con 0 exec-timeout 0 0 logging synchronous line aux 0 line vty 0 4 password cisco login ! end |

# EIGRP Routing tables:

ISP Router (EIGRP)

Codes: C - connected, S - static, I - IGRP, R - RIP, M - mobile, B - BGP D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2 E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2 ia - IS-IS inter area, * - candidate default, U - per-user static route o - ODR, P - periodic downloaded static route Gateway of last resort is 207.62.41.26 to network 0.0.0.0 172.16.0.0/16 is variably subnetted, 2 subnets, 2 masks D 172.16.0.128/26 [90/2172416] via 207.62.41.22, 02:27:16, Serial0/3 D 172.16.0.0/25 [90/2172416] via 207.62.41.26, 02:27:02, Serial0/1 207.62.41.0/24 is variably subnetted, 7 subnets, 2 masks C 207.62.41.24/30 is directly connected, Serial0/1 C 207.62.41.26/32 is directly connected, Serial0/1 D 207.62.41.28/30 [90/21024000] via 207.62.41.26, 02:29:52, Serial0/1 [90/21024000] via 207.62.41.22, 02:29:52, Serial0/3 D 207.62.41.29/32 [90/21024000] via 207.62.41.26, 02:29:56, Serial0/1 D 207.62.41.30/32 [90/21024000] via 207.62.41.22, 02:29:52, Serial0/3 C 207.62.41.20/30 is directly connected, Serial0/3 C 207.62.41.22/32 is directly connected, Serial0/3 63.0.0.0/8 is variably subnetted, 2 subnets, 2 masks C 63.88.27.64/30 is directly connected, Serial0/0 C 63.88.27.128/25 is directly connected, FastEthernet0/0 D*EX 0.0.0.0/0 [170/2297856] via 207.62.41.26, 02:29:58, Serial0/1

# BR_1 Router (EIGRP)

# Codes: C - connected, S - static, I - IGRP, R - RIP, M - mobile, B - BGP # D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area # N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2 # E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP # i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2 # ia - IS-IS inter area, * - candidate default, U - per-user static route # o - ODR, P - periodic downloaded static route # Gateway of last resort is 207.62.41.30 to network 0.0.0.0 # 172.16.0.0/16 is variably subnetted, 2 subnets, 2 masks # C 172.16.0.128/26 is directly connected, FastEthernet0/0 # D 172.16.0.0/25 [90/20514560] via 207.62.41.30, 02:31:28, Serial0/0 # 207.62.41.0/24 is variably subnetted, 7 subnets, 2 masks # D 207.62.41.24/30 [90/21024000] via 207.62.41.30, 02:34:17, Serial0/0 # [90/21024000] via 207.62.41.21, 02:34:17, Serial0/1 # D 207.62.41.25/32 [90/21024000] via 207.62.41.30, 02:34:19, Serial0/0 # D 207.62.41.26/32 [90/21024000] via 207.62.41.21, 02:34:17, Serial0/1 # C 207.62.41.28/30 is directly connected, Serial0/0 # C 207.62.41.30/32 is directly connected, Serial0/0 # C 207.62.41.20/30 is directly connected, Serial0/1 # C 207.62.41.21/32 is directly connected, Serial0/1 # 63.0.0.0/8 is variably subnetted, 2 subnets, 2 masks # D EX 63.88.27.64/30 [170/21024000] via 207.62.41.21, 02:34:19, Serial0/1 # D 63.88.27.128/25 [90/20514560] via 207.62.41.21, 02:31:15, Serial0/1 # D*EX 0.0.0.0/0 [170/20640000] via 207.62.41.30, 02:34:19, Serial0/0

# BR_2 Router (EIGRP)

# Codes: C - connected, S - static, I - IGRP, R - RIP, M - mobile, B - BGP # D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area # N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2 # E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP # i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area # * - candidate default, U - per-user static route, o - ODR # P - periodic downloaded static route # Gateway of last resort is 0.0.0.0 to network 0.0.0.0 # 172.16.0.0/16 is variably subnetted, 2 subnets, 2 masks # D 172.16.0.128/26 [90/20514560] via 207.62.41.29, 00:02:54, Serial0/0 # C 172.16.0.0/25 is directly connected, FastEthernet0/0 # 207.62.41.0/24 is variably subnetted, 7 subnets, 2 masks # C 207.62.41.24/30 is directly connected, Serial0/1 # C 207.62.41.25/32 is directly connected, Serial0/1 # C 207.62.41.28/30 is directly connected, Serial0/0 # C 207.62.41.29/32 is directly connected, Serial0/0 # D 207.62.41.20/30 [90/21024000] via 207.62.41.25, 00:02:55, Serial0/1 # [90/21024000] via 207.62.41.29, 00:02:55, Serial0/0 # D 207.62.41.21/32 [90/21024000] via 207.62.41.29, 00:02:55, Serial0/0 # D 207.62.41.22/32 [90/21024000] via 207.62.41.25, 00:02:57, Serial0/1 # 10.0.0.0/30 is subnetted, 1 subnets # C 10.10.10.8 is directly connected, Loopback100 # 63.0.0.0/25 is subnetted, 1 subnets # D 63.88.27.128 [90/20514560] via 207.62.41.25, 00:02:55, Serial0/1 # S* 0.0.0.0/0 is directly connected, Loopback100

Command Summary:

| # Command | # Description |
| --- | --- |
| #delete flash:vlan.dat | Clears the ‘VLAN’ data stored in flash |
| #erase startup-config | Erase any previous running configs stored in memory |
| #reload | Restarts the IOS after removing any stored startup-configs |
| #clear mac-address-table secure | Clears any previously stored MAC addresses |
| #clear port-security dynamic | Clears previously stored port-security data |
| #vlan database | Assign VLAN database to Cisco 2900XL |
| #vtp client | Assigns switch status to ‘client’ – Propagates data w/ ‘server’ |
| #vtp server | Create, delete, modify VLANS in ‘vtp server mode’ |
| #vlan 20 name marketing | Assings the name ‘marketing’ to VLAN 20 |
| #vtp domain corporate | Assigns domain name ‘corporate’. |
| #vtp password cisco | Assigns VLAN database password to ‘cisco’ |
| (config)#int fa 0/7 (config-if)#switchport mode access (config-if)#port security max 1 (config-if)#switchport access vlan 20 | Configure interface port #7 of Cisco 2900XL switch Assigns port #7 as ‘access mode’ on 2900XL switch Assigns maximum 1 mac address to port #7 on the switch Assigns the ‘access port #7’ to ‘VLAN 20’ |
| (config)#int vlan1 (config-if)#ip add 192.168.10.3 255.255.255.224 | Assigns ip address and subnet mask to ‘VLAN 1’ |
| (config-if)#spanning-tree vlan 1 root primary | Assigns 2950 as root switch and enables ‘spanning tree’ |
| (config-if)#switchport port-security | Enables port security |
| (config-if)#switchport port-security maximum 1 | Sets maximum MAC addresses = 1 |
| (config-if)#switchport port-security violation shutdown | If port exceeds 1 MAC address, then action is to ‘shutdown’ |
| #show port_security | Shows port security messages |
| #ip dhcp excluded-address 192.168.10.65 | Address cannot be issued during ‘DHCP’ |
| #ip dhcp pool vlan1 #network 192.168.10.0 255.255.255.224 #default-router 192.168.10.1 | Assigns dhcp to VLAN 1 Assigns a range of ip addresses for VLAN 1 Assigns default gateway |
| (config)#int fa0/0.1 #encaps dot1q 1 native #ip add 192.168.10.1 255.255.255.224 #ip nat inside | Configure interface fast Ethernet 0/0.1 Enables the interface to accept 802.1Q VLAN packets Assigns ip address and subnet mask for interface Enables Network Address Translation on incoming packets |
| (config)#ip nat inside source list 1 int s0/0 overload #access-list 1 permit 192.168.10.0 0.0.0.31 | Provide Access Lists for specified networks used for NAT Lists available ip address for a given network |
| (config)#int s0/0 (config-if)#ip address 63.88.27.66 255.255.255.252 (config-if)#ip nat outside (config-if)#encaps frame-relay (config-if)#frame-relay map ip 63.88.27.65 102 | Configure interface ‘serial 0/0’ Assign ip address and subnet mask to serial port ‘s0/0’ Enable Network Address Translation on outgoing packets Encapsulation used = frame relay Map frame relay packets to DLCI 102 => 63.88.27.65 |
| (config)#int s0/1 (config-if)#ip add 207.62.41.25 255.255.255.252 (config-if)#encaps ppp (config-if)#ppp authentication chap | Configure Serial interface 0/1 of ISP Assign ip address and subnet mask for S0/1 Use encapsulation PPP (Point to Point Protocol) Use ‘chap’ |
| (config)#router ospf 10 (config-router)#network 207.62.41.20 0.0.0.3 area 0 | Use OSPF Enter networks and reverse subnet mask w/ area number |
| Br-2(config)#int loopback100 (config-if)#ip add 10.10.10.10. 255.255.255.252 | Configure Loopback interface ‘100’ Assign ip address and subnet mask to interface |
| (config)#ip route 0.0.0.0 0.0.0.0 63.88.27.65 | Next hop address when going through frame relay |

Troubleshooting/Reflection:

Problem: Static routes not being distributed between isp, br_1 and br_2.

Resolution:

a) Static routes can be defined with a local interface or the next hop IP address. We HAD TO SPECIFY the next hop instead of the interface on both sides of the frame relay before isp router's static route was distributed by eigrp to br_1 and br_2.

b) In addition Gary recommended we use "redistribute connections" on isp's EIGRP configuration to properly handle the frame-relay correctly.

Problem: Switches could not ping router.

Resolution: Forgot to "no shut" the vlan 1 interface on both switches which caused ping to fail on these devices.This command was added to the copy/paste command file to prevent happening again.

Problem: Switches could not ping the router

Resolution: A cross-over cable was initially connected between the distribution switch and the corporate router which caused dhcp to fail. Installing the correct straight through cable fixed this. Only one cross-over cable is needed and that is between the two switches.Revised the wiring diagram to better clarify which cables should be used to prevent this from happening again.

Problem:Incorrect routing tables on isp, br_1 and br_2.

Resolution: Typos were uncovered by visual inspections after the network did not converge. Most of the typos were network and IP addresses.

Problem:Hosts below the cloud could not ping the router or get dhcp addresses.

Resolution: The Max 1 port security configured on access switch was blocking ports 4, 7 and 10. The fix was to clear the mac addresses that were added to the running config with "no ..."'s.A simpler method was found later which is issue the command "clear mac-address-table secure" on access (2900XL) and "clear port-security dynamic" on the distribution (2950). These commands were added to the copy/paste config files to prevent it from reoccurring.

Problem: Unable to type any commands into TeraTerm serial interface to Corporate.

Resolution: Keyboard cable was not plugged in all the way into the back of the PC. Was dislodged by USB memory stick. Worked fine when pushed back in.

Problem: Istanbul host PC did not power up.

Resolution: The PCs have two power switches. The one on the back of the PC, on the power supply itselt, was switched off. Switching it back on solved the problem.

Problem: EIGRP error messages on adjacent neighbors.

Resolution: Added a "passive-interface fa 0/0" to the EIGRP section of ISP, br_1 and br_2. They were all connected into VLAN 1 ports on Distribution and even though they were configured as separate subnets the routing information was getting passed through the switch. The passive-interface command fixed this problem.

Problem: OSPF didn't like the no auto-summary command.

Resolution: Didn't use it and it still converged on Netlab and Cape Town.

Problem: OSPF did not distribute static route on ISP to br_1 and br_2

Resolution: Remove static route on ISP to Corporate and add that network into the OSPF networks instead. Also removed the redistribute static and redistribute connections from ISP. This worked and routing tables on br_1 and br_2 now included a route to the 63.88.27.64 network.

Test Plan:

The test methodology involved creating test cases for each Case Study requirement. For convergence this included end-to-end ping tests and verifying routing tables had routes to all networks. For NAT/PAT this included pinging both directions (outside to inside should fail) and using debug ip icmp to make sure address translation was happening. For DHCP testing we configured three hosts with DHCP on different corporate VLANs and then checked the assigned IP address for being in the proper subnet. To check VLSM we looked at the routing tables. We also check DHCP bindings on corporate. We used several commands on the switches to verify security and vlan configuration. Also we found one switch security defect with a peer review of running-configs. The test cases are documented in Appendix A.

Appendix A

Test Plan

Here is our test plan to make sure that everything is working.

BTC = Below the cloud

OTC = Over the cloud

BTC-1) Verify RFC 1918, VLSM, DHCP

On corporate

show ip route

verify we have four 192.168.x.x subnets and multiple subnet masks.

verify default gateway configured

save copy for the report

BTC-2) Verify DHCP

On host PCs

disable/enable network connections on Yellow. Purple and Green hosts

- Yellow is on VLAN 10 and should get IP 192.168.10.34+ and GW 192.168.10.33

- Purple is on VLAN 20 and should get IP 192.168.10.66+ and GW 192.168.10.65

- Green  is on VLAN 99 and should get IP 192.168.10.82+ and GW 192.168.10.81

On corporate

show ip dhcp binding

BTC-3) Verify end-to-end convergence and NAT

On isp

should fail:

ping 192.168.10.1

ping 192.168.10.2

ping 192.168.10.3

ping 192.168.10.65

ping 192.168.10.81

debug ip icmp

all pings tests (below) from corporate LAN should only use 63.88.27.66

Yellow PC

!ping corporate ports and sub-ifs

ping 192.168.10.1

ping 192.168.10.33

ping 192.168.10.65

ping 192.168.10.81

ping 63.88.27.66

!ping isp ports

ping 63.88.27.65

ping 63.88.27.129

ping 207.62.41.21

ping 207.62.41.25

!ping isp host

ping 63.88.27.130

!ping br_1 ports

ping 172.16.0.129

ping 207.62.41.22

ping 207.62.41.29

!ping br_1 host

ping 172.16.0.130

!ping br_2 ports

ping 172.16.0.1

ping 207.62.41.30

ping 207.62.41.26

ping 10.10.10.10

traceroute 10.10.10.10

!ping br_2 host

ping 172.16.0.2

Purple PC

!ping corporate ports and sub-ifs

ping 192.168.10.1

ping 192.168.10.33

ping 192.168.10.65

ping 192.168.10.81

ping 63.88.27.66

!ping isp ports

ping 63.88.27.65

ping 63.88.27.129

ping 207.62.41.21

ping 207.62.41.25

!ping isp host

ping 63.88.27.130

!ping br_1 ports

ping 172.16.0.129

ping 207.62.41.22

ping 207.62.41.29

!ping br_1 host

ping 172.16.0.130

!ping br_2 ports

ping 172.16.0.1

ping 207.62.41.30

ping 207.62.41.26

ping 10.10.10.10

traceroute 10.10.10.10

!ping br_2 host

ping 172.16.0.2

Green PC

!ping corporate ports and sub-ifs

ping 192.168.10.1

ping 192.168.10.33

ping 192.168.10.65

ping 192.168.10.81

ping 63.88.27.66

!ping isp ports

ping 63.88.27.65

ping 63.88.27.129

ping 207.62.41.21

ping 207.62.41.25

!ping isp host

ping 63.88.27.130

!ping br_1 ports

ping 172.16.0.129

ping 207.62.41.22

ping 207.62.41.29

!ping br_1 host

ping 172.16.0.130

!ping br_2 ports

ping 172.16.0.1

ping 207.62.41.30

ping 207.62.41.26

ping 10.10.10.10

traceroute 10.10.10.10

!ping br_2 host

ping 172.16.0.2

On corporate

show ip nat translation

show ip nat statistics

- save copy for report

BTC-4) Switch configuration

On both switches

!ping isp ports

ping 63.88.27.65

ping 63.88.27.129

ping 207.62.41.21

ping 207.62.41.25

!ping isp host

ping 63.88.27.130

ping 207.62.41.25

!ping br_1 ports

ping 172.16.0.129

ping 207.62.41.22

ping 207.62.41.29

!ping br_1 host

ping 172.16.0.130

!ping br_2 ports

ping 172.16.0.1

ping 207.62.41.30

ping 207.62.41.26

ping 10.10.10.10

traceroute 10.10.10.10

!ping br_2 host

ping 172.16.0.2

On Access

show vlan

- Verify each switch has at least one port on VLAN 1, 10, 20 and 99

- save copy for report

show run

- verify each port manually configured as manual or trunk port

- verify each access port has Max 1 security configured

- save copy for report

show spanning-tree

- verify access is not root switch

- save copy for report

show int fa 0/1 switchport

- verify trunking

- save copy for report

On Distribution

show vlan

- Verify each switch has at least one port on VLAN 1, 10, 20 and 99

- save copy for report

show run

- verify each port manually configured as manual or trunk port

- verify each access port has Max 1 security configured

- save copy for report

show spanning-tree

- verify distribution is the root switch

- save copy for report

show int gi 0/1 switchport

- verify trunking

- save copy for report

show int gi 0/2 switchport

- verify trunking

- save copy for report

BTC-5) Be able to telnet to all switches

On corporate

telnet 192.168.10.2

login and exit

telnet 192.168.10.3

login and exit

OTC-6) Convergence tests between top three routers

!ping isp ports

ping 63.88.27.65

ping 63.88.27.129

ping 207.62.41.21

ping 207.62.41.25

!ping isp host

ping 63.88.27.130

!ping br_1 ports

ping 172.16.0.129

ping 207.62.41.22

ping 207.62.41.29

!ping br_1 host

ping 172.16.0.130

!ping br_2 ports

ping 172.16.0.1

ping 207.62.41.30

ping 207.62.41.26

ping 10.10.10.10

traceroute 10.10.10.10

!ping br_2 host

ping 172.16.0.2

on isp, br_1 and br_2 verify ip routes for:

172.16.0.128/26

172.16.0.0/25

207.62.41.24/30

207.62.41.28/30

207.62.41.20/30

10.10.10.8

63.88.27.64/30

63.88.27.128/25

Default gateway (of last resort) is set

on br_2

telnet to isp, br_1, corporate and capture running configs

telnet to both switches (via corporate) and capture running configs

Appendix B

Case Study Requirements

Here we show the case study requirements and how we met each requirement.

### Corporate LAN

1.  IP Addressing

## Use RFC 1918

Use VLSM

We subnetted using the following:

VLAN 1 	192.168.10.0/27

VLAN 10 	192.168.10.32/27

VLAN 20	192.168.10.64/28

VLAN 99	192.168.10.80/28

2.  Access and Distribution Switches

- Configure ports as a member of VLAN 1, 10, 20, 99.  Each switch must have at least one port in each VLAN.

Distribution Switch

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

1    default                          active    Fa0/1, Fa0/2, Fa0/3, Fa0/4

Fa0/5, Fa0/6

10   accounting                       active    Fa0/7, Fa0/8, Fa0/9, Fa0/10

Fa0/11, Fa0/12

20   marketing                        active    Fa0/13, Fa0/14, Fa0/15, Fa0/16

Fa0/17, Fa0/18

99   engineering                      active    Fa0/19, Fa0/20, Fa0/21, Fa0/22

Fa0/23, Fa0/24

Access Switch

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

1    default                          active    Fa0/2, Fa0/3

10   accounting                       active    Fa0/4, Fa0/5, Fa0/6

20   marketing                        active    Fa0/7, Fa0/8, Fa0/9

99   engineering                      active    Fa0/10, Fa0/11, Fa0/12

- All ports configured manually as access or trunk ports.

Distribution Switch – sample

interface FastEthernet0/24

switchport access vlan 99

switchport mode access

switchport port-security

no ip address

!

interface GigabitEthernet0/1

switchport mode trunk

no ip address

Access Switch – sample

interface FastEthernet0/1

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface FastEthernet0/4

port security max-mac-count 1

switchport access vlan 10

- Port security configured on all access ports to allow only 1 MAC address.

Distribution Switch – sample

distribution#show port-security

Secure Port  MaxSecureAddr  CurrentAddr  SecurityViolation  Security Action

(Count)       (Count)          (Count)

---------------------------------------------------------------------------

Fa0/1              1            0                  0         Shutdown

Fa0/2              1            0                  0         Shutdown

Fa0/3              1            0                  0         Shutdown

# Access Switch – sample

access#show run

interface FastEthernet0/2

port security max-mac-count 1

!

interface FastEthernet0/3

port security max-mac-count 1

- STP Root Bridge is the Distribution switch

distribution#show spanning-tree

VLAN0001

Spanning tree enabled protocol ieee

Root ID    Priority    24577

Address     000b.fc28.d400

This bridge is the root

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

3.  Configure trunking to allow for all VLANs to intercommunicate.

distribution#show vtp status

VTP Version                     : 2

Configuration Revision          : 1

Maximum VLANs supported locally : 250

Number of existing VLANs        : 8

VTP Operating Mode              : Server

VTP Domain Name                 : corporate

Access#show vtp status

VTP Version                     : 2

Configuration Revision          : 1

Maximum VLANs supported locally : 68

Number of existing VLANs        : 8

VTP Operating Mode              : Client

VTP Domain Name                 : corporate

corporate#show ip route

Codes: C - connected, S - static, I - IGRP, R - RIP, M - mobile, B - BGP

192.168.10.0/24 is variably subnetted, 4 subnets, 2 masks

C       192.168.10.64/28 is directly connected, FastEthernet0/0.20

C       192.168.10.80/28 is directly connected, FastEthernet0/0.99

C       192.168.10.32/27 is directly connected, FastEthernet0/0.10

C       192.168.10.0/27 is directly connected, FastEthernet0/0.1

63.0.0.0/30 is subnetted, 1 subnets

C       63.88.27.64 is directly connected, Serial0/0

S*   0.0.0.0/0 [1/0] via 63.88.27.65

4.  Corporate Router

- Default Gateway for Corporate LAN

corporate#show ip route

Gateway of last resort is 63.88.27.65 to network 0.0.0.0

63.0.0.0/30 is subnetted, 1 subnets

C       63.88.27.64 is directly connected, Serial0/0

S*   0.0.0.0/0 [1/0] via 63.88.27.65

- DHCP Server for Corporate LAN

corporate#show run

ip dhcp excluded-address 192.168.10.1 192.168.10.3

ip dhcp excluded-address 192.168.10.33

ip dhcp excluded-address 192.168.10.65

ip dhcp excluded-address 192.168.10.81

!

ip dhcp pool vlan1

network 192.168.10.0 255.255.255.224

default-router 192.168.10.1

!

ip dhcp pool vlan10

network 192.168.10.32 255.255.255.224

default-router 192.168.10.33

!

ip dhcp pool vlan20

network 192.168.10.64 255.255.255.240

default-router 192.168.10.65

!

ip dhcp pool vlan99

network 192.168.10.80 255.255.255.240

default-router 192.168.10.81

- NAT/PAT for Corporate LAN

corporate#show run

interface FastEthernet0/0.1

encapsulation dot1Q 1 native

ip address 192.168.10.1 255.255.255.224

ip nat inside

!

interface FastEthernet0/0.10

encapsulation dot1Q 10

ip address 192.168.10.33 255.255.255.224

ip nat inside

!

interface FastEthernet0/0.20

encapsulation dot1Q 20

ip address 192.168.10.65 255.255.255.240

ip nat inside

!

interface FastEthernet0/0.99

encapsulation dot1Q 99

ip address 192.168.10.81 255.255.255.240

ip nat inside

!

interface Serial0/0

ip address 63.88.27.66 255.255.255.252

ip nat outside

ip nat inside source list 1 interface Serial0/0 overload

corporate#show ip nat statistics

Total active translations: 0 (0 static, 0 dynamic; 0 extended)

Outside interfaces:

Serial0/0

Inside interfaces:

FastEthernet0/0.1, FastEthernet0/0.10, FastEthernet0/0.20

FastEthernet0/0.99

Hits: 89  Misses: 35

Expired translations: 35

Dynamic mappings:

-- Inside Source

[Id: 1] access-list 1 interface Serial0/0 refcount 0

isp#debug ip icmp

ICMP packet debugging is on

isp#

02:54:17: ICMP: echo reply sent, src 63.88.27.65, dst 63.88.27.66

isp#

02:54:42: ICMP: echo reply sent, src 63.88.27.129, dst 63.88.27.66

isp#

02:55:14: ICMP: echo reply sent, src 207.62.41.21, dst 63.88.27.66

isp#

02:55:25: ICMP: echo reply sent, src 207.62.41.25, dst 63.88.27.66

- Connected to ISP Router using Frame Relay

a.  DLCI 102

b.  Use Frame Relay Map statement

c.  Default Route to ISP (use next hop address, issue with Adtran)

d.  /30 Subnet

corporate#show run

interface Serial0/0

ip address 63.88.27.66 255.255.255.252

ip nat outside

encapsulation frame-relay

frame-relay map ip 63.88.27.65 102

corporate#show ip route

63.0.0.0/30 is subnetted, 1 subnets

C       63.88.27.64 is directly connected, Serial0/0

S*   0.0.0.0/0 [1/0] via 63.88.27.65

corporate#show frame-relay map

Serial0/0 (up): ip 63.88.27.65 dlci 102(0x66,0x1860), static,

CISCO, status defined, active

5.  Other

- Ability to remotely manage all switches (telnet).

corporate#telnet 192.168.10.2

Trying 192.168.10.2 ... Open

User Access Verification

Password:

distribution>en

Password:

distribution#copy run tftp

Address or name of remote host []? 63.88.27.130

Destination filename [distribution-confg]?

!!

2766 bytes copied in 3.792 secs (922 bytes/sec)

distribution#

corporate#telnet 192.168.10.3

Trying 192.168.10.3 ... Open

User Access Verification

Password:

access>en

Password:

access#copy run tftp

Address or name of remote host []? 63.88.27.130

Destination filename [running-config]?

!!

1801 bytes copied in 3.874 secs (600 bytes/sec)

### Network

1.  ISP Router

- Static route to Corporate LAN

isp#sh ip route

63.0.0.0/8 is variably subnetted, 2 subnets, 2 masks

C       63.88.27.64/30 is directly connected, Serial0/0

C       63.88.27.128/25 is directly connected, FastEthernet0/0

O*E2 0.0.0.0/0 [110/1] via 207.62.41.26, 01:05:14, Serial0/1

isp#show run

ip route 63.88.27.64 255.255.255.252 63.88.27.66

- Connected to ISP Router using Frame Relay

  - DLCI 201

  - Use Frame Relay Map statement

  - Default Route to ISP (use next hop address, issue with Adtran)

  - /30 Subnet

isp#show run

interface Serial0/0

ip address 63.88.27.65 255.255.255.252

encapsulation frame-relay

frame-relay map ip 63.88.27.66 201

frame-relay lmi-type ansi

isp#sh ip route

63.0.0.0/8 is variably subnetted, 2 subnets, 2 masks

C       63.88.27.64/30 is directly connected, Serial0/0

C       63.88.27.128/25 is directly connected, FastEthernet0/0

O*E2 0.0.0.0/0 [110/1] via 207.62.41.26, 01:05:14, Serial0/1

isp#show frame-relay map

Serial0/0 (up): ip 63.88.27.66 dlci 201(0xC9,0x3090), static,

CISCO, status defined, active

2.  ISP, Branch Office 1, Branch Office 2

- Serial Connections

  - PPP

  - Chap Authentication

  - /30 Subnets

ISP Router

isp#show run

interface Serial0/1

ip address 207.62.41.25 255.255.255.252

encapsulation ppp

ppp authentication chap

!

interface Serial0/3

ip address 207.62.41.21 255.255.255.252

encapsulation ppp

ppp authentication chap

BR_1 Router

BR_1#show run

interface Serial0/0

ip address 207.62.41.29 255.255.255.252

encapsulation ppp

no fair-queue

ppp authentication chap

!

interface Serial0/1

ip address 207.62.41.22 255.255.255.252

encapsulation ppp

clockrate 64000

ppp authentication chap

BR_2 Router

BR_2#show run

interface Serial0/0

ip address 207.62.41.30 255.255.255.252

encapsulation ppp

clockrate 64000

ppp authentication chap

!

interface Serial0/1

ip address 207.62.41.26 255.255.255.252

encapsulation ppp

clockrate 64000

ppp authentication chap

3.  Routing Protocol

- EIGRP or OSPF

We did both.  Both running configurations & routing tables are included in the main document.

- Default traffic sent to Branch Office 2, Loopback 100

br_2#sh ip int brief

Interface                  IP-Address      OK? Method Status           Protocol

FastEthernet0/0            172.16.0.1      YES manual up               up

Serial0/0                  207.62.41.30    YES manual up               up

Serial0/1                  207.62.41.26    YES manual up               up

Loopback100                10.10.10.10     YES manual up               up

br_2#sh ip route

S*   0.0.0.0/0 is directly connected, Loopback100

br_1#sh ip route

Gateway of last resort is 207.62.41.30 to network 0.0.0.0

O*E2 0.0.0.0/0 [110/1] via 207.62.41.30, 01:00:31, Serial0/0

4.  Configure LAN interfaces for Branch Office 1 and Branch Office 2

- Branch Office 1 and Branch Office 2 are VLSM  networks, part of 172.16.0.0

br_1#sh ip route

172.16.0.0/16 is variably subnetted, 2 subnets, 2 masks

C       172.16.0.128/26 is directly connected, FastEthernet0/0

O       172.16.0.0/25 [110/782] via 207.62.41.30, 01:00:31, Serial0/0

br_2#sh ip route

172.16.0.0/16 is variably subnetted, 2 subnets, 2 masks

O       172.16.0.128/26 [110/782] via 207.62.41.29, 00:53:04, Serial0/0

C       172.16.0.0/25 is directly connected, FastEthernet0/0

### Verification

1.  Workstation on Corporate LAN should be able to ping all other interfaces in network.

Sample

C:\Documents and Settings\cisco>ping 10.10.10.10

Pinging 10.10.10.10 with 32 bytes of data:

Reply from 10.10.10.10: bytes=32 time=49ms TTL=253

Reply from 10.10.10.10: bytes=32 time=48ms TTL=253

Reply from 10.10.10.10: bytes=32 time=51ms TTL=253

Reply from 10.10.10.10: bytes=32 time=49ms TTL=253

Ping statistics for 10.10.10.10:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss),

Approximate round trip times in milli-seconds:

Minimum = 48ms, Maximum = 51ms, Average = 49ms

C:\Documents and Settings\cisco>ping 172.16.0.2

Pinging 172.16.0.2 with 32 bytes of data:

Reply from 172.16.0.2: bytes=32 time=58ms TTL=125

Reply from 172.16.0.2: bytes=32 time=51ms TTL=125

Reply from 172.16.0.2: bytes=32 time=49ms TTL=125

Reply from 172.16.0.2: bytes=32 time=48ms TTL=125

C:\Documents and Settings\cisco>ping 207.62.41.26

Pinging 207.62.41.26 with 32 bytes of data:

Reply from 207.62.41.26: bytes=32 time=51ms TTL=253

Reply from 207.62.41.26: bytes=32 time=49ms TTL=253

Reply from 207.62.41.26: bytes=32 time=48ms TTL=253

Reply from 207.62.41.26: bytes=32 time=46ms TTL=253

C:\Documents and Settings\cisco>ping 172.16.0.130

Pinging 172.16.0.130 with 32 bytes of data:

Reply from 172.16.0.130: bytes=32 time=55ms TTL=125

Reply from 172.16.0.130: bytes=32 time=48ms TTL=125

Reply from 172.16.0.130: bytes=32 time=51ms TTL=125

Reply from 172.16.0.130: bytes=32 time=50ms TTL=125

C:\Documents and Settings\cisco>ping 63.88.27.130

Pinging 63.88.27.130 with 32 bytes of data:

Reply from 63.88.27.130: bytes=32 time=37ms TTL=126

Reply from 63.88.27.130: bytes=32 time=35ms TTL=126

Reply from 63.88.27.130: bytes=32 time=34ms TTL=126

Reply from 63.88.27.130: bytes=32 time=32ms TTL=126

C:\Documents and Settings\cisco>ping 192.168.10.81

Pinging 192.168.10.81 with 32 bytes of data:

Reply from 192.168.10.81: bytes=32 time=1ms TTL=255

Reply from 192.168.10.81: bytes=32 time=1ms TTL=255

Reply from 192.168.10.81: bytes=32 time=1ms TTL=255

Reply from 192.168.10.81: bytes=32 time=1ms TTL=255